Table of Contents

# Revised Manuscript Sections — SPE Journal (Data Science and Engineering Analytics)

Companion to the referee report. Every quantitative statement below is taken from the submitted manuscript or from the authors’ own result files. No new results have been invented. Where a claim in the original could not be supported by the evidence available, it has been removed or requalified rather than restated — those points are flagged in 〔EDITOR’S NOTE〕 boxes.

Citations are given in SPE author-date style, as required by the SPE Technical Publications Style Guide (2022). [CITATION NEEDED] marks a claim that requires a source the current reference list does not contain.

## 0. Title

The submitted title names the framework after its least-evaluated component. Three alternatives, in order of recommendation:

- Safety-Relevant Evaluation of Edge-Deployable Machine Learning for Gas-Hazard Monitoring: Accuracy Rankings Do Not Predict Missed-Hazard Behavior

- Beyond Classification Accuracy: Missed-Hazard, False-Alarm, and Degradation Metrics for Machine-Learning Gas Detection on Low-Cost Edge Hardware

- Cost-Sensitive Action Assignment for Gas-Hazard Monitoring at the Edge: A Leakage-Controlled Benchmark Study

Recommendation 1 states the finding rather than the apparatus, which is the convention in strong DSEA papers and is the framing the evidence supports.

## 1. Summary

〔EDITOR’S NOTE〕 SPE Journal heads this section “Summary,” not “Abstract.” The version below leads with the operational problem, states the dataset’s nature honestly, attaches confidence bounds to every zero-count claim, reports the negative results, and removes the α = 0.05 significance claim (which is unattainable by construction at n = 5) and the word “uniquely.” It is 297 words.

Fixed gas detection at wellpads, tank batteries, compressor stations, and gas-processing facilities generates far more abnormal indications than an operator can act on. The engineering difficulty is rarely the absence of a signal; it is deciding, under degraded and ambiguous evidence, which indication warrants which response. Machine-learning studies of gas detection are almost universally ranked by classification accuracy, a metric that is silent on the two quantities that determine whether a monitoring system is safe to deploy: how often a hazard draws no response, and how often clean conditions draw a high-severity one.

We evaluate an edge-deployable monitoring pipeline — thermal-image classification, normal-only sensor anomaly scoring, cost-sensitive action assignment, and locally generated natural-language explanation — under a safety-relevant metric set rather than accuracy alone. Evaluation uses the public MultimodalGasData corpus (Narkhede et al. 2022) with a leakage-controlled block-wise holdout and a 20-window embargo. The corpus contains laboratory surrogates (incense smoke, alcohol-based vapor, and their mixture), not hydrocarbons; results therefore characterize evaluation methodology rather than field performance.

Under this protocol, eight models spanning classical and deep architectures fall within 3.3 accuracy points of one another (0.944–0.977), yet separate by two orders of magnitude on out-of-distribution missed-hazard rate: withholding one hazardous class at training time, a supervised multilayer perceptron misses 18.7% of unseen hazardous windows (95% Clopper–Pearson upper bound 20.4%) while cost-weighted and boosted models miss none (bound 0.19%). Escalation behavior separates further and in the opposite direction: on a second unseen hazardous class, gradient boosting escalates 100% of hazardous windows to alarm-grade action, whereas the cost-weighted policy escalates 29.8 ± 8.6%, with the shortfall invisible to the miss metric. Cost asymmetry, swept from 1:1 to 20:1, changes no observed miss rate and degrades accuracy above 6:1. On a Raspberry Pi 4, decoupling explanation generation from the decision path and applying INT8 quantization reduce the deployable path from 14.8 s to 235 ms, at the cost of one high-severity false alarm in 316 clean windows (bound 1.49%).

The practical implication is that accuracy-ranked model selection is uninformative about safety behavior, and that missed-hazard rate alone is insufficient: escalation adequacy, false-alarm burden, degradation under sensor loss, and confidence bounds on all zero counts must be reported together.

Keywords: gas-hazard monitoring; safety-relevant evaluation; cost-sensitive learning; distribution shift; edge deployment; alarm management; anomaly detection; explainable AI

## 2. Introduction

〔EDITOR’S NOTE〕 The rewrite makes four changes. (i) It lands the problem in an oil and gas facility with a named asset class, as SPE DSEA reviewers expect. (ii) It replaces the “these capabilities are usually treated separately” gap — integration is apparatus, not contribution — with a gap in evidence. (iii) It states the paper’s negative results in the Introduction rather than deferring them. (iv) It removes the reinforcement-learning framing throughout.

Major process-safety incidents are seldom preceded by silence. More often the warning is present, distributed across several indications, and never converted into timely and proportionate action. In the eleven minutes before the Milford Haven refinery explosion, the control-room operator received 275 alarms (Goel et al. 2017). The failure was not sensing. It was the conversion of sensed evidence into a decision.

This distinction is sharp at upstream and midstream facilities. Wellpads, tank batteries, compressor stations, and gas-processing plants carry fixed point-gas detectors — commonly low-cost metal-oxide-semiconductor (MOX) elements — whose indications feed alarm systems already operating near or beyond the alarm-rate envelopes recommended by industry guidance [CITATION NEEDED: EEMUA 191; ISA-18.2]. The same facilities face increasing obligation to detect and act on fugitive hydrocarbon release, where the response is simultaneously a safety action and an emissions action. In both cases the operational question is not what gas is present but what should be done now, and how confident must we be to do it.

Machine learning has been applied extensively to the first question and scarcely to the second. Gas-detection and classification models are ranked by classification accuracy, and on established benchmarks that ranking has saturated: reported accuracies on the corpus used here span 93–99.7% across more than ten peer-reviewed studies (Narkhede et al. 2021; Sharma et al. 2024; Zhang and Zhang 2025) [CITATION NEEDED: complete the enumeration]. When a benchmark saturates, differences between the leading methods stop being informative about deployment, and the interesting question becomes what the accuracy ranking is failing to measure.

### 2.1 Problem statement

Accuracy is a symmetric metric applied to an asymmetric problem. In a monitoring context, the error classes are not exchangeable:

- A hazardous condition that draws no response forfeits the intervention window entirely.

- A hazardous condition that draws a sub-alarm response — increased sampling, a request for verification — preserves some of that window but not all of it, and is not equivalent to a correct escalation.

- A clean condition that draws a high-severity response consumes operator attention, and, at sufficient rate, degrades the alarm system that later hazards depend on (Laberge et al. 2014).

- A hazardous condition that draws the wrong high-severity response is, in most operational regimes, nearly as good as the right one.

No single scalar orders these. A model ranked first on accuracy may be ranked last on any of them, and nothing in an accuracy table reveals which.

### 2.2 What is known and what is not

The component technologies are mature. Reconstruction-based models trained on normal operation detect departures from baseline in multivariate sensor streams (Malhotra et al. 2016), including in field-validated pipeline settings (Liang et al. 2024). Vision models classify thermal and optical gas signatures at embedded-device speeds (Korjani et al. 2024). Cost-sensitive learning provides a principled account of unequal error costs (Elkan 2001) [CITATION NEEDED]. Compact language models make local explanation feasible without transmitting monitoring data off-site.

What is not established is how these components behave when ranked by anything other than accuracy, and in particular:

- whether accuracy-ranked model selection preserves the ranking under a missed-hazard criterion;

- whether models that agree on in-distribution data continue to agree when the hazard is one they were never trained on — the condition that matters most, since a deployed detector will eventually meet a release it has not seen;

- whether missed-hazard rate is itself sufficient, or whether it can be satisfied by under-escalation that a miss count does not register;

- whether these behaviors survive the arithmetic and thermal constraints of the low-cost edge hardware on which such systems are actually installed.

This paper addresses these four questions.

### 2.3 Contributions

- A safety-relevant evaluation protocol for gas-monitoring models, comprising: a leakage-controlled block-wise holdout with an explicit embargo; a five-level action target that separates missed hazard from under-escalation from high-severity false alarm; leave-one-class-out (LOCO) evaluation against an unseen hazard; a graded perturbation suite; and Clopper–Pearson bounds on every zero count. To our knowledge this combination has not been applied to any MOX gas-detection benchmark.

- Evidence that accuracy ranking is uninformative about safety behavior. Eight models spanning classical and deep architectures span 2.6 accuracy points on the block-wise holdout but separate by a factor of ~100 in out-of-distribution missed-hazard rate. The highest accuracy in our comparison belongs to a random forest, not to any of the deep models.

- Evidence that missed-hazard rate is itself insufficient. On an unseen hazardous class, models with statistically indistinguishable miss rates differ by 3.4× in the proportion of hazardous windows escalated to alarm-grade action. The model with the better miss rate has the worse escalation rate. A safety metric set that reports misses alone will select the wrong model.

- Two negative results reported in full. Cost asymmetry, swept from 1:1 to 20:1, produces no measurable change in miss rate on this corpus and degrades accuracy above 6:1; and a deterministic label-function baseline attains the accuracy ceiling by construction, establishing that the benefit of a learned policy here, if any, lies entirely in degraded conditions rather than in nominal ones.

- An edge-deployment characterization on a Raspberry Pi 4, quantifying the latency, footprint, quantization, and sustained-thermal costs of running the full pipeline within a 2 s sampling budget, including the false-alarm cost of INT8 quantization.

〔EDITOR’S NOTE〕 Contribution 1 of the original (“modular monitoring pipeline of five coordinated components”) has been removed. Architecture is apparatus. Original contribution 2 (asymmetric reward) has been converted into negative result 4, because the authors’ own cost-ratio sweep shows no effect on the miss rate at any ratio.

## 3. Methods — targeted replacement passages

Only the passages requiring substantive correction are given. Numbering follows the submitted manuscript.

### 3.1 Dataset and preprocessing — replacement for paragraphs 1 and 3

Replacement paragraph 1.

Evaluation uses the public MultimodalGasData corpus (Narkhede et al. 2022; Mendeley Data, CC BY 4.0), which pairs a seven-channel MQ-series MOX array with synchronized 206×156 thermal frames from a Seek Compact camera. The corpus comprises 6,400 labeled samples in four classes of 1,600 samples each: clean air (NoGas), incense smoke, alcohol-based deodorant vapor (Perfume), and a smoke–vapor mixture. The analytes are laboratory surrogates, not hydrocarbons. They exercise the MOX transduction mechanism, cross-sensitivity structure, and drift behavior that a facility detector would encounter, but they do not establish detection performance for methane or heavier hydrocarbons. Results in this paper should be read as characterizing evaluation methodology on a representative MOX corpus, not as field performance. The four classes were recorded as four contiguous blocks within a single acquisition session of approximately 90 minutes. Within-session baseline drift is therefore confounded with class identity — the condition shown by Dennler et al. (2022) to inflate reported accuracy on comparably structured MOX benchmarks and to have affected a substantial body of prior published work [CITATION NEEDED]. The split design in the following paragraph is a direct response to this property of the corpus.

Replacement paragraph 3 (split).

Seven-channel sensor readings were formed into sliding windows of length 20. Consecutive windows share 19 of 20 raw rows, so a random train–test split places near-duplicate windows in both partitions; a plain sequential split is equally unsuitable because the corpus is block-ordered by class. We therefore use a leakage-controlled block-wise holdout: within each class block, the final 20% of sequential windows form the test partition and the earlier windows form the training partition, with a temporal gap of G = 20 windows discarded between them so that no training window shares a raw row with any test window. Normalization parameters were estimated on the training partition only. This yields 5,024 training and 1,276 test windows (316 NoGas; 320 each of Smoke, Mixture, and Perfume).

〔EDITOR’S NOTE — BLOCKING〕 Three window counts appear across the manuscript and the authors’ result files: 6,380 (§3.1 arithmetic and §4.3), 6,324 (REPORT.md, “after dropping class-boundary-straddling windows”), and 1,581 per class implied by the Clopper–Pearson bounds in Table 8. These must be reconciled to one number and one stated procedure, and the text must say explicitly whether windows straddling a class boundary were retained or discarded. The 316/320/320/320 asymmetry should be explained in the same sentence.

〔EDITOR’S NOTE — BLOCKING〕 The split is generated once and is not reseeded across runs. The following sentence must be added, because it changes how every ± in the paper is read: “The block-wise partition is fixed across all runs; reported standard deviations therefore reflect model-initialization variance and exclude data-partition variance, which is generally the larger of the two in small-sample comparison (Bouthillier et al. 2021).” [CITATION NEEDED]

### 3.2 System architecture — paragraph to insert

Insert after the four design principles.

The framework is advisory. It occupies the alarm-management and operator-support layer and does not actuate. The action labeled Emergency Shutdown denotes a recommendation that the operator initiate the facility’s existing emergency shutdown procedure; it does not command a trip. The safety instrumented function remains an independent, deterministic, separately verified layer, consistent with the independence requirement of layer-of-protection analysis and with the absence of any established certification pathway for a learned component within a safety instrumented function under IEC 61511 [CITATION NEEDED]. All results in this paper are obtained with the framework in this advisory position, and the facility’s protective function is unaffected by its removal.

### 3.3 Decision Agent — full replacement

〔EDITOR’S NOTE — BLOCKING, W1〕 The submitted §3.3 and §4.4 describe reinforcement learning: an asymmetric reward with four components, ε-greedy action selection, 200 episodes of replay, cumulative reward, and a Huber loss. The authors’ own retrain/results/REPORT.md states that the model producing every reported number is trained by “cost-weighted cross-entropy against rule-oracle action.” That is supervised classification with per-sample loss weights: no reward is evaluated, no policy is improved, no return is bootstrapped. The replacement below describes the model that produced the results. Figure 7 and the “Training dynamics” subsection of §4.4 must be deleted with it. This correction is not optional and should carry explicit co-author sign-off.

Decision module. The decision module maps the 22-dimensional monitoring state φ_t to one of five operator-facing actions (Table 3). It is a feedforward network with a dueling value–advantage head, trained by cost-weighted cross-entropy on the training partition. Target actions are generated by a deterministic map from the ground-truth class to its acceptable action set: NoGas→{0}, Smoke→{3}, Mixture→{4}, Perfume→{1, 2}. Each training sample carries a scalar loss weight w(g_t) encoding the asymmetry of consequence: w = c_miss for hazardous classes (Smoke, Mixture) and w = c_false otherwise, with the cost ratio C = c_miss / c_false. The objective is

L(θ) = − (1/N) Σ_t w(g_t) · log p_θ(a*(g_t) | φ_t) ………………..(1)

where a*(·) is the target action map, p_θ is the softmax over the five action scores, and N is the number of training samples. The deployed configuration uses C = 8:1. Section 4.x reports the full sweep C ∈ {1:1, …, 20:1}.

〔EDITOR’S NOTE〕 Two consequences of this formulation must be stated in the text, because a reviewer will derive them independently:

- The target action is a deterministic function of the class label. Decision accuracy is therefore four-class gas classification accuracy composed with a fixed map, not a distinct quantity. The paper’s argument must be reframed accordingly: what differs between the decision framing and the classification framing is not the task but the error metric — which is a real and defensible contribution, and is what the LOCO and perturbation results actually demonstrate.

- The deployed C = 8:1 is not the accuracy optimum (C = 6:1 is, at 0.9658) and was selected on test-set variance. Either re-select on a validation split or state the selection criterion explicitly and note the confound.

### 3.3 Anomaly Detection Agent — passage to correct

〔EDITOR’S NOTE — BLOCKING, W5〕 Table 5 reports 79 false positives on 1,580 normal windows — exactly 5.000%, the nominal value of a 95th-percentile threshold evaluated on the same sample that defined it. The stated procedure (τ fitted on the NoGas training partition, evaluated on all normal windows) would not produce an exact 5.000%. Either the text mis-describes the procedure or the evaluation is in-sample; the reported ROC-AUC of 0.963 is in any case computed over all 6,380 windows, including those the autoencoder was trained on. Refit τ on a disjoint calibration split, re-evaluate on held-out NoGas windows only, and re-report AUC, TPR, and FPR.

Additionally: because the anomaly score is state feature 0 and is consumed by the framework’s escalation logic, the manuscript must report the per-class anomaly-score distribution and state whether the score ranks hazard monotonically. If it does not, the feature is not performing the function the text assigns it, and this must be said.

### 3.3 Perception Agent — passage to correct

〔EDITOR’S NOTE — BLOCKING, W6〕 The sentence “The image-level split is separate from the sensor-window split described in Section 3.1 and does not create the same temporal leakage risk” asserts without argument the opposite of what the acquisition protocol implies. The thermal frames are consecutive captures from the same ~90-minute session as the sensor rows; adjacent frames are near-duplicates, and a stratified random 80/20 split over them places near-identical images in both partitions. This is the exact failure mode the authors spend a 20-window embargo avoiding on the sensor modality. Re-split the images block-wise and re-report. If accuracy falls, report the fall: it is direct evidence for the paper’s own argument about evaluation protocol, and it strengthens rather than weakens the contribution. Until this is done, 98.8% top-1 must be reported as an optimistic upper bound, not a performance estimate.

## 4. Results — reframed passages

### 4.5 Model comparison — replacement text

Eight models — the cost-weighted policy, an unweighted network of identical topology, a multilayer perceptron, gradient boosting, a support vector classifier, a random forest, a recurrent network over raw windows, and a conservative offline-RL baseline — were trained on the same training partition with the same five initialization seeds and evaluated on the same block-wise holdout (Table 7).

All eight fall within 3.3 accuracy points of one another (0.9437–0.9769). The highest point accuracy belongs to the random forest (0.9769 ± 0.0086), which also has the lowest variance; the cost-weighted policy places fourth (0.9633 ± 0.0215). Seven of eight record no danger miss on the clean holdout; gradient boosting records 0.0098 ± 0.0219. No model records a high-severity false alarm.

We assert no ranking among these models, and the design does not support one. With n = 5 paired seeds, the two-sided Wilcoxon signed-rank test has a minimum attainable p-value of 2^(1−n) = 0.0625; no comparison in this design can reach α = 0.05 at any effect size. The smallest observed p-value is 0.1875 (random forest). Because the block-wise partition is fixed across runs, the reported standard deviations additionally exclude data-partition variance. The appropriate reading of Table 7 is that these eight models are not distinguishable on in-distribution accuracy under this protocol — which is the premise for everything that follows, and is more useful than a ranking would be.

〔EDITOR’S NOTE — BLOCKING, W4〕 The claim “no baseline significantly outperforms ARXIS at α = 0.05” must be deleted from the Summary, §4.5, §5.3, and §7. It is true, unfalsifiable under this design, and — since the random forest has the higher point estimate — rhetorically reversed. If the authors wish to make a positive claim of equivalence, the correct instrument is a Bayesian correlated t-test with a region of practical equivalence (Benavoli et al. 2017) over ≥29 randomized runs varying seed, split, augmentation, and hyperparameter draw (Bouthillier et al. 2021). That would license the far stronger sentence: “the cost-weighted policy and the classical baselines are practically equivalent on this benchmark, with posterior probability p.” [CITATION NEEDED for both]

〔EDITOR’S NOTE〕 Table 7’s 0.9633 and the paper’s headline 94.44% are two values for the same system on the same holdout; the manuscript never reconciles them, nor the MLP’s 0.9616 (Table 7) against 98.20% (Table 9). Reconcile or explain the protocol difference in the text.

### 4.6 Leave-one-class-out evaluation — replacement section

This is the paper’s principal experiment and should be presented as such, with a figure of its own.

For each held-out class, all models were retrained on the remaining three classes and evaluated on the withheld one. Held-out accuracy is 0.0 in every cell by construction: withholding a class removes its target action from the training label set, so the model cannot emit it. The informative quantities are therefore what the model does instead — whether it fails safe, and how far it escalates.

Table 8 (revised). Leave-one-class-out results on held-out hazardous classes. Danger-miss counts only the assignment of action 0. Escalation is the proportion of hazardous windows assigned an alarm-grade action (a ≥ 3). n = 1,581 hazardous windows per cell; bounds are 95% one-sided Clopper–Pearson.

| Held-out | Model | Danger-miss | 95% CP bound | Escalation (a ≥ 3) |
| --- | --- | --- | --- | --- |
| Smoke | Cost-weighted policy | 0.0000 | ≤ 0.19% | 1.000 ± 0.000 |
| Smoke | Unweighted network | 0.0000 | ≤ 0.19% | 1.000 ± 0.000 |
| Smoke | Gradient boosting | 0.0000 | ≤ 0.19% | 1.000 ± 0.000 |
| Smoke | Multilayer perceptron | 0.1875 | ≤ 20.41% | 0.811 ± 0.137 |
| Mixture | Cost-weighted policy | 0.0043 | ≤ 0.83% | 0.298 ± 0.086 |
| Mixture | Unweighted network | 0.0000 | ≤ 0.19% | 0.249 ± 0.123 |
| Mixture | Multilayer perceptron | 0.0035 | ≤ 0.75% | 0.289 ± 0.142 |
| Mixture | Gradient boosting | 0.0000 | ≤ 0.19% | 1.000 ± 0.000 |

Two findings follow, and they point in opposite directions.

First, in-distribution accuracy does not predict out-of-distribution missed-hazard rate. On held-out Smoke, the multilayer perceptron — statistically indistinguishable from every other model on the clean holdout — assigns the passive Monitor action to 18.7% of unseen hazardous windows (296 of 1,581 in the reference seed; 95% CP bound ≤20.41%), a rate two orders of magnitude above the ≤0.19% bound attained by the other three. Nothing in Table 7 anticipates this.

Second, and more consequential for how such systems should be evaluated: missed-hazard rate is itself insufficient. On held-out Mixture, all four models record miss rates below 0.5%, yet three of them escalate only about a quarter to a third of unseen hazardous windows to alarm-grade action. The remaining ~70% receive Increase Sampling or Request Verification — a sub-alarm response to the most hazardous class in the corpus, which the miss metric does not register because it counts only the assignment of action 0. Gradient boosting, which is not a cost-weighted model, escalates 100% of the same windows. On the quantity that determines whether an operator is notified, the simplest baseline in this comparison outperforms the cost-weighted policy by a factor of 3.4.

We report this because it is the more useful result. A metric set that reports missed-hazard rate alone would select the cost-weighted policy over gradient boosting on this evidence, and would be wrong to. Escalation adequacy must be reported alongside miss rate, and a hazardous window answered with a request for verification must not be scored as a success.

〔EDITOR’S NOTE — BLOCKING, W3〕 The escalation column exists in the authors’ retrain/results/exp3_loco.csv and is absent from the submitted manuscript, while §5.3 asserts that the framework “uniquely preserves zero danger misses under distribution shift.” That assertion is not supportable and must be removed. Reporting the escalation column is both the correct action and the better paper. The remaining four models (SVM, random forest, recurrent network, CQL) should be added to this experiment or their absence explained. The provenance caveat recorded in NOTE_provenance_exp3.md — that the LOCO grid was executed as independent per-class runs owing to a platform process limit — belongs in a reproducibility statement.

### 4.7 Cost-asymmetry sweep — section to add

〔EDITOR’S NOTE〕 This experiment exists in retrain/results/exp1_pareto.csv and does not appear in the manuscript. It is the direct test of the paper’s original second contribution and it returns a negative result. Reporting it is necessary; it is also, in our judgment, one of the more valuable paragraphs available to this paper.

The loss-weight ratio C was swept across {1:1, 2:1, 4:1, 6:1, 8:1, 10:1, 12:1, 16:1, 20:1}, five seeds each, with all else held fixed.

The observed danger-miss rate is zero at every ratio, including the symmetric 1:1 baseline. Accuracy rises modestly to a maximum of 0.9658 at 6:1, is 0.9633 at the deployed 8:1, and falls monotonically thereafter to 0.8396 at 20:1. No high-severity false alarm is recorded at any ratio.

Cost asymmetry therefore produces no measurable safety benefit on this corpus, and beyond 6:1 it costs accuracy — at 20:1, twelve points of it. We attribute this to the structure of the problem rather than to the mechanism: because the target action is a deterministic function of a well-separated class label, and because a miss requires assigning a hazardous window to the class furthest from it, the miss rate is already at the floor before any weighting is applied. The correct inference is not that cost asymmetry is ineffective in general — the cost-sensitive learning literature is unambiguous that it is effective where the error surface is nontrivial (Elkan 2001) [CITATION NEEDED] — but that this benchmark cannot demonstrate it, and that demonstrating it requires a corpus, or a metric, in which sub-alarm responses carry cost. The LOCO escalation result in Section 4.6 is exactly such a metric, and there the cost-weighted policy does not lead.

### 4.7 Robustness — text to revise

〔EDITOR’S NOTE — W12〕 Table 9 reports accuracy and danger-miss under perturbation but not false-alarm or mean action severity. Under three-channel dropout, accuracy falls to 29.9% while danger misses remain zero — a combination that is equally consistent with graceful degradation and with a policy that has begun escalating indiscriminately. The reader cannot distinguish them. Add a false-alarm-rate column and a mean-action-severity column to every perturbation row. Both are computable from existing runs.

Further: the rule-oracle row (100.00%) is the label function evaluated on itself, an identity rather than a measurement, and it requires the true class label so it is not deployable. Label it as such, and add a deployable comparator — a hand-tuned threshold rule on the MQ channels — to every row including the perturbed ones. Without an incumbent-practice baseline under perturbation, the paper’s central robustness claim has nothing to be robust relative to.

Finally, the perturbation intensities are unmotivated. Adopt a graded severity structure (five levels per perturbation type) and report a breakdown point per model — the severity at which safety behavior first fails. Channel dropout is the most operationally realistic perturbation for a fixed detector and warrants the full k = 1…7 sweep.

Revised interpretive paragraph. Under calibration drift of ±40%, the supervised perceptron retains higher decision accuracy than the cost-weighted policy (59.1% vs. 53.7%) while introducing a danger-miss rate of 0.0187; the cost-weighted policy records none. This is the clearest illustration in our results of why accuracy and safety behavior must be reported separately: the more accurate model is the less safe one, and no accuracy table would reveal it. Under channel dropout the ordering reverses on accuracy as well — with one channel removed the policy retains 55.6% against the perceptron’s 36.0% — while both preserve zero misses. We note that at these accuracy levels the models are no longer performing the task in any useful sense, and that “zero danger misses” at 29.9% accuracy should be read as a statement about failure mode, not about fitness for service.

### 4.8 Calibration — replacement text

Expected calibration error was computed for four uncertainty estimators (Table 10). The deep ensemble attains the lowest mean ECE (0.0123) and is the only estimator whose error is stable across partitions (± 0.0043). For the remaining three, the standard deviation exceeds the differences between them: raw softmax 0.0489 ± 0.0699, MC dropout 0.0533 ± 0.0692, temperature scaling 0.0319 ± 0.0397. No ranking among those three is supportable from these data.

The instability is itself the finding. An ECE that varies by a factor of five across initializations of the same model on the same fixed partition is not a property of the calibration method; it is evidence that single-model confidence on this corpus is partition-sensitive to a degree that would matter operationally, since a confidence estimate is what an escalation threshold would be set against. Ensembling removes the instability at five times the inference cost — a trade-off that the edge-deployment budget in Section 4.9 makes non-trivial and that should be stated as such.

〔EDITOR’S NOTE〕 Add: bootstrap confidence intervals, a bin-count sensitivity sweep, class-conditional ECE (the hazardous class is the one an escalation threshold acts on), multiclass Brier score and negative log-likelihood, and reliability diagrams. Also note that the reported p = 0.03125 for temperature scaling and for the deep ensemble is the minimum attainable one-sided p-value at n = 5 and means only “better on all five seeds”; it carries no information about effect size.

## 5. Discussion

〔EDITOR’S NOTE〕 The submitted Discussion restates results. This rewrite interprets them, states what changes operationally, addresses the objections a reviewer will raise, and does not claim anything the evidence does not carry. The unsupported ablation claim has been removed.

### 5.1 Accuracy ranking is not a safety ranking

The central empirical result of this study is a dissociation. Under a leakage-controlled protocol, eight model families spanning tree ensembles, kernel methods, feedforward networks, recurrent networks, and offline-RL variants are indistinguishable on in-distribution accuracy — 3.3 points separate the best from the worst, and the design cannot resolve differences of that size. Under distribution shift the same models separate by two orders of magnitude on missed-hazard rate, and by a further factor of 3.4 on escalation adequacy. Nothing in the accuracy table predicts either separation, and the ordering is not preserved: the model with the best out-of-distribution miss rate on one held-out hazard is among the worst on escalation for another, and the highest in-distribution accuracy belongs to a random forest that appears in no part of the safety argument.

The practical consequence is direct. A team selecting a gas-monitoring model on benchmark accuracy — which is what the published literature on this corpus supports them in doing — has no information about how the selected model will behave on a release it was not trained on. That is the condition under which the model will eventually be asked to perform.

### 5.2 Missed-hazard rate is necessary and not sufficient

The second result is a caution about the obvious remedy. Having established that accuracy is the wrong metric, the natural substitute is missed-hazard rate. Our leave-one-class-out results show this to be insufficient in a specific and consequential way.

A missed-hazard metric that counts only the fully passive response scores a hazardous window answered with Request Verification as a success. On held-out Mixture, three of four models satisfy the miss metric almost perfectly (≤0.83% at 95% confidence) while escalating only about a quarter to a third of hazardous windows to alarm-grade action. An operator running such a system would receive no alarm for roughly seven in ten windows of the most hazardous class in the corpus, and the safety metric would report success.

We therefore propose that safety-relevant evaluation of monitoring models report, at minimum: missed-hazard rate with a confidence bound; escalation adequacy against the class-conditional target severity; high-severity false-alarm rate expressed as alarm burden per operator per unit time; degradation of all three under graded sensor loss and calibration drift; and the sample size supporting each zero count. Each of these is cheap to compute and each of them changed a conclusion in this study.

### 5.3 What the asymmetric objective did and did not do

Our cost-ratio sweep returns no measurable effect on miss rate at any ratio from 1:1 to 20:1, and an accuracy penalty above 6:1. We take this seriously rather than setting it aside.

The reason is structural. Because the target action is a deterministic function of a well-separated class label, and because a missed hazard requires assigning a hazardous window to the class most distant from it in feature space, the miss rate sits at the floor before any weighting is applied. There is no error surface for the asymmetry to reshape. This is a property of the benchmark, not a refutation of cost-sensitive learning — but it has a general implication worth stating: on a saturated, well-separated benchmark, a safety objective cannot be validated, because the unsafe behavior it exists to suppress does not occur. Demonstrating that a cost asymmetry works requires either a corpus in which hazardous and benign states overlap, or a metric under which sub-alarm responses carry cost. Section 4.6 supplies the second, and under it the cost-weighted policy does not lead.

### 5.4 Deployment on low-cost edge hardware

Three deployment findings hold across configurations.

Explanation must be decoupled from the decision path. Synchronous local language-model explanation produces a mean end-to-end latency of 14,738 ms against a 2,000 ms sampling budget — a factor of seven over budget, on hardware that is otherwise entirely adequate. Moving explanation off the critical path and quantizing to INT8 reduces the deployable path to 235 ms (P99 413 ms). The architectural conclusion generalizes beyond this system: in a monitoring context, a language model’s role is to explain a decision already taken, never to participate in taking it, and the latency arithmetic enforces what the safety argument already requires.

Quantization has a measurable safety cost. INT8 halves the footprint (8.74 → 4.19 MB) and introduces one high-severity false alarm across 316 clean windows (0.0032; 95% CP upper bound 1.49%). Whether that is acceptable is a facility decision, not a modeling one. Where it is not, the decision network can remain in floating point while the larger perception model is quantized — the decision network is not the source of the footprint.

Sustained thermal behavior narrows the margin. Over a 36-minute passively cooled run the processor reaches its 80°C throttle point after approximately 24 minutes, reducing clock from 1,500 to 1,234 MHz and raising median latency from 420 to 640 ms. The decision path remains inside budget, but the margin is halved by an effect that a short benchmark would not reveal. Deployment qualification for this class of hardware should include a sustained thermal run.

### 5.5 Position relative to prior work on this corpus

Reported classification accuracies on MultimodalGasData span 93–99.7%, and the decision accuracy reported here (94.44%) sits below most of them. We state this plainly because the comparison is not like-for-like in either direction, and both directions matter.

Against us: our evaluation is on a leakage-controlled block-wise holdout with an explicit embargo, while the published comparisons are, to our knowledge, computed on shuffled splits over overlapping windows drawn from a single recording session. Dennler et al. (2022) showed that this protocol conflates recording time with class label on a comparably structured MOX corpus and flagged a substantial body of published work for re-evaluation [CITATION NEEDED]. The accuracies are therefore not directly comparable, and the direction of the bias is known.

In our favor: our numbers measure a different quantity. A classification error and an action error carry different operational consequence — confusing two hazardous classes preserves the need to escalate, whereas assigning a hazardous window to Monitor forfeits the intervention window. For a like-for-like recognition comparison the relevant figure is the thermal classifier’s accuracy, subject to the split caveat in Section 3.3.

We do not claim to advance the accuracy state of the art on this corpus, and we do not think that would be a useful thing to advance. The benchmark is saturated; the open question is what the saturated numbers fail to measure.

### 5.6 Limitations and threats to validity

Analyte validity. The corpus contains incense smoke, alcohol-based vapor, and their mixture. These are surrogates. No result here establishes detection performance for methane or heavier hydrocarbons, and external validity to petroleum facility monitoring is argued from the shared MOX transduction mechanism rather than demonstrated. Work crossing from laboratory MOX corpora to controlled-release field data has reported accuracy falling by several points [CITATION NEEDED]; we expect at least that.

Single-session provenance. All 6,400 samples come from one acquisition session on one device. Sensor ageing, inter-device variability, seasonal ambient variation, wind, and long-term drift are unrepresented. Cross-session and cross-device validation is the single most important outstanding experiment.

Fixed partition. The block-wise split is generated once. Reported standard deviations capture initialization variance only and exclude partition variance.

Statistical power. Five paired seeds cannot resolve differences at α = 0.05 under a Wilcoxon signed-rank test. No ranking among the eight models is asserted, and none should be inferred.

Metric scope. Danger-miss counts only the fully passive action. Section 4.6 shows that this understates under-response, and we report escalation adequacy alongside it for that reason; a graded expected-cost formulation would be preferable and is not attempted here.

Modality scope. The action policy consumes sensor dynamics and the anomaly score only. Visual evidence informs the perception and explanation paths but not the action. Figure 2 should be read accordingly.

Evaluation of the coordination layer. The components that coordinate, critique, and supervise the pipeline are described as apparatus and are not experimentally isolated. No claim in this paper depends on them, and none should be read as validating them.

Configuration selection. The deployed cost ratio (8:1) was chosen on test-partition variance rather than on a held-out validation split.

## 6. Conclusions

〔EDITOR’S NOTE〕 The submitted Conclusions repeat the Abstract, including both unsupported claims. Conclusions should state what is now known at the level of the field. This version asserts nothing the evidence does not carry.

We evaluated an edge-deployable gas-monitoring pipeline under a safety-relevant metric set rather than accuracy alone, using a leakage-controlled block-wise holdout on a public MOX benchmark whose analytes are laboratory surrogates.

- Accuracy ranking carries no information about safety behavior. Eight model families span 3.3 accuracy points on the block-wise holdout — a range the design cannot resolve — yet separate by two orders of magnitude in missed-hazard rate on an unseen hazardous class (18.7%, 95% CP bound ≤20.41%, versus ≤0.19%). Model selection on benchmark accuracy is therefore uninformative for this application.

- Missed-hazard rate is necessary but not sufficient. On a second unseen hazardous class, models with miss rates below 0.83% at 95% confidence escalate between 24.9% and 100% of hazardous windows to alarm-grade action. The shortfall is invisible to a miss metric that counts only the fully passive response, and the model with the best miss rate is not the model with the best escalation rate. Escalation adequacy must be reported alongside miss rate.

- A cost-asymmetric objective produced no measurable safety benefit on this benchmark, at any ratio from 1:1 to 20:1, and degraded accuracy above 6:1. Where hazardous and benign states are well separated and the target action is a deterministic function of the class, there is no error surface for a safety objective to reshape. Validating such an objective requires a corpus with genuine overlap, or a metric under which under-escalation carries cost.

- Full local operation is achievable within a 2 s sampling budget on commodity edge hardware, provided that language-model explanation is decoupled from the decision path: 14,738 ms synchronous falls to 235 ms asynchronous with INT8 quantization, at a cost of one high-severity false alarm in 316 clean windows (≤1.49%), with a sustained-thermal margin that halves after 24 minutes of passive-cooled operation.

Taken together, these results argue that missed-hazard behavior, escalation adequacy, false-alarm burden, degradation under sensor loss and drift, and confidence bounds on all zero counts should be reported as standard alongside accuracy in machine-learning studies of gas-hazard monitoring. On the evidence here, a selection made on accuracy alone would have been uninformative about every one of these behaviors.

Priorities for further work are, in order: cross-session and cross-device validation; evaluation on a corpus containing a controlled hydrocarbon release; class-conditional conformal prediction to convert observed zero-miss counts into distribution-free coverage guarantees on the hazardous class; and comparison against classical sequential change detection, which addresses the same escalation problem with provable run-length guarantees and is the natural incumbent for this application.

## 7. Nomenclature

〔EDITOR’S NOTE〕 SPE expects a Nomenclature section. The manuscript has none. Draft below; complete once Eq. 1’s notation is finalized.

| Symbol | Definition |
| --- | --- |
| a | safety action, a ∈ {0, 1, 2, 3, 4} |
| a*(·) | acceptable target-action map from gas class to action set |
| C | cost ratio, C = c_miss / c_false |
| c_miss, c_false | loss weights for hazardous and non-hazardous training samples |
| E_t | operator-facing natural-language explanation at cycle t |
| G | embargo length between training and test partitions, in windows |
| g_t | ground-truth gas class for window t |
| I_t | thermal image at cycle t |
| L(θ) | cost-weighted cross-entropy objective, Eq. 1 |
| N | number of training samples |
| p_θ | action probability distribution under model parameters θ |
| X_t | seven-channel sensor window at cycle t, X_t ∈ ℝ^(20×7) |
| X’_t | normalized sensor window |
| δ_t | per-sensor change across the window |
| θ | model parameters |
| ρ̃_t | normalized reconstruction-error anomaly score |
| σ_t | per-sensor standard deviation across the window |
| τ | anomaly detection threshold |
| φ_t | 22-dimensional decision state, φ_t ∈ ℝ²² |

Subscripts: t — inference cycle or window index. Abbreviations to define at first use: CP (Clopper–Pearson); ECE (expected calibration error); ESD (emergency shutdown); LOCO (leave-one-class-out); MOX (metal-oxide semiconductor); ROC-AUC (area under the receiver operating characteristic curve); SIS (safety instrumented system); VOC (volatile organic compound).

## 8. References — conversion notes

SPE requires author-date in-text citation and an alphabetized reference list with SPE paper numbers and DOI hyperlinks. Convert all 26 entries. Specific corrections:

| Ref | Correction |
| --- | --- |
| [6] Liang et al. | Published in SPE Production & Operations 38 (3): 513, not SPE Journal. Add SPE paper number and DOI. [CITATION NEEDED — verify] |
| [8] Mnih et al. | Mnih, V., et al. is malformed under SPE style — list up to three authors, then et al. Same for [13], [21], [25]. |
| [14] Gemma | Cited as 2024 but used for Gemma 3 1B, a later release. Cite the correct technical report and year. [CITATION NEEDED] |
| [18] YOLOv7 | Cited to support a YOLO-family claim while the system uses YOLOv8 [22]. Cite [22] directly or make the family claim explicitly historical. |
| [19]/[20] Narkhede | Cite the 2022 Data descriptor as the dataset of record, with its Mendeley DOI, and acknowledge the CC BY 4.0 licence. |
| [26] Zhang and Zhang | Described as a “Transformer classifier” in Table 12; the reference title says “optimized deep forest classifier.” Fix the method description. |
| — | Add: Dennler et al. (2022) on MOX benchmark drift; Elkan (2001) on cost-sensitive learning; Bouthillier et al. (2021) and Benavoli et al. (2017) on ML comparison methodology; Clopper and Pearson (1934); EEMUA 191; ISA-18.2; IEC 61511. [CITATION NEEDED for all] |

## 9. Figure budget

Eleven figures are currently allocated, six of them (Figs. 3, 4, 5, 6, 8, 9) to two component models, while the paper’s principal experiment has none. Suggested reallocation:

| Action | Figure |
| --- | --- |
| Delete | Fig. 7 (training dynamics of a run that produced no reported result) |
| Merge | Figs. 3 + 4 → one perception panel; Figs. 5 + 6 → one anomaly panel; Figs. 8 + 9 → one policy-behavior panel |
| Add | LOCO miss rate and escalation rate by held-out class and model — the paper’s headline |
| Add | Cost-ratio sweep: accuracy and miss rate vs. C, with the flat miss line made visible |
| Add | Perturbation breakdown curves: miss rate, escalation, and false alarm vs. severity, all models |
| Keep | Figs. 1, 2 (redraw 2 so the vision branch’s advisory-only path is explicit), 10, 11 |